OBN Project — Conclusion Summary

Ultrasonic Object Discrimination · 2-Session Study · RF / SVM / KNN

Frankfurt University of Applied Sciences · Machine Learning WS 2025/26

Romana Rashid (1428733)  ·  Mithila Maruti Prabhu (1567111)

1. ACCURACY SCORECARD

| Best model 82.4% Random Forest · Day 1 in-session | Cross-day drop 35.3% Random Forest · Day 2 full range | Best cross-day 51.8% RF · 4-distance retrained model |
| --- | --- | --- |

The 5-fold cross-validation mean for RF on Day 1 is 72.0 ± 4.9%, which is 10 pp below the single-split figure — indicating optimistic bias. SVM CV: 57.8 ± 3.3%. KNN CV: 61.1 ± 3.2%.

2. CLASSIFIER COMPARISON · DAY 2 FULL RANGE

| Classifier | Accuracy (Day 2 full range) | Score |
| --- | --- | --- |
| Random Forest |  | 35.3% |
| KNN (k=7, dist-weighted) |  | 32.3% |
| SVM (RBF, C=10) |  | 31.3% |
| Random chance baseline = 25% (4 classes equally likely) |  |  |

3. CLASS DIFFICULTY · RF DAY 2 F1

| Class | F1 (RF full) | Note |
| --- | --- | --- |
| Steel plate | 0.40 | → 0.76 with 4-dist RF |
| Laptop | 0.41 |  |
| PB + foam | 0.28 |  |
| PB no foam | 0.33 | hardest class |

Why Steel is easiest: High acoustic impedance produces a strong, spectrally narrow echo that remains distinctive after drift. RF 4-dist F1: 0.76.

Why PB no foam is hardest: Hollow plastic front face is nearly identical to PB+foam. argmax values overlap across classes on Day 2. Low SNR at longer distances.

4. ROOT CAUSE OF DRIFT — FEATURE INTER-SESSION SHIFT

The table below compares the percentage mean shift of key features between Day 1 and Day 2, per class. Red rows are the primary drift drivers; green is the stable alternative.

| Feature | Laptop | PB+foam | PB−foam | Steel |
| --- | --- | --- | --- | --- |
| argmax (sample index) | +78.6% | +569.5% | +34.2% | +85.4% |
| argmin (sample index) | +76.9% | +369.6% | +36.4% | +87.7% |
| fft_peak_bin | +0.3% | +0.8% | +0.0% | +0.8% |
| abs_mean | +8.7% | +14.6% | +15.3% | +5.3% |
| fft_max | +10.7% | +14.4% | +16.8% | +12.1% |

Key insight: argmax encodes absolute sample index within the 8,192-sample ADC window. A sensor repositioning of even a few mm shifts the echo peak by hundreds of samples, inverting the Day 1 class ordering completely. fft_peak_bin encodes the transducer resonance frequency and is therefore session-stable.

Day 1 argmax class means:  Steel 1620 → Laptop 1275 → PB−foam 710 → PB+foam 108

Day 2 argmax class means:  PB−foam 952 → PB+foam 724 → Laptop 273 → Steel 236  (order reversed)

5. FEATURE REDUNDANCY

| 38 feature pairs with ¦r¦ > 0.95 out of 105 possible pairs | 4 perfectly collinear pairs (r = 1.000) std ↔ rms std ↔ fft_std rms ↔ fft_std energy ↔ fft_energy |
| --- | --- |

The four perfectly collinear pairs arise from: (1) RMS = std when signal mean ≈ 0; (2) Parseval's theorem links time-domain energy to FFT energy. The effective independent dimensionality of the 16-feature vector is only 4–5 signals.

6. OTHER KEY FINDINGS

| Majority vote (per file) RF: 35.3% → 39.4% (+4.1 pp) Day 2 has 104 source files (26 per class). Taking the modal prediction across 5 measurements per file gives a modest but consistent gain for RF. SVM and KNN gain less. | Confidence calibration failure RF gap: 0.048 · SVM gap: 0.014 · KNN gap: 0.090 Correct vs. wrong predictions receive nearly identical confidence scores. Rejection thresholds provide almost no benefit under cross-day drift. |
| --- | --- |

7. RECOMMENDATIONS

| Tier 1 | Remove argmax / argmin — address root cause Replace with fft_peak_bin (drifts ≤0.84% vs 570% for argmax), Hilbert-envelope time-of-flight, spectral centroid, and zero-crossing rate. Exploit the currently unused FPGA FFT block embedded in each raw data file. |
| --- | --- |
| Tier 2 | Cut redundant features & improve evaluation Drop rms, fft_std, fft_energy (r=1.000 duplicates of existing features). Adopt leave-one-session-out cross-validation as the primary metric. Compare XGBoost and MLP classifiers. |
| Tier 3 | Domain adaptation if retraining is not feasible Apply CORAL covariance alignment to shift Day 2 feature distributions toward the Day 1 training distribution before inference — requires no labelled Day 2 data. |

These results confirm that ultrasonic classifiers trained in a single session are not deployment-ready without periodic retraining or explicit drift mitigation. Removing argmax/argmin and adopting leave-one-session-out validation are the two highest-priority changes.